STC89C52 MICROCONTROLLER

Register and Memory Documentation

| Document Item | Details |
| --- | --- |
| Microcontroller | STC89C52RC / 8051 family |
| Contribution | Register and Memory Documentation |
| Purpose | Reference for the simulator's CPU, memory and instruction-execution modules |
| Prepared by | Niranjana S |
| Week | Week 1 |

# 1. Contribution Scope

This document provides a structured study of the registers and memory organization of the STC89C52RC microcontroller. The information is intended to support the team's educational microcontroller simulator, especially the parts that display CPU state, execute instructions, access memory, manage the stack, and represent special function registers (SFRs).

Document the main CPU registers and their functions.

Document the internal RAM organization and register banks.

Document the Special Function Register (SFR) address space.

Explain stack operation and the Stack Pointer.

Explain program memory versus data memory.

Identify bit-addressable memory and bit-addressable SFRs.

Provide simulator-oriented guidance for representing registers and memory.

# 2. STC89C52RC Memory Overview

The STC89C52RC is an enhanced 80C51-compatible microcontroller. The device provides 8 KB of on-chip Flash program memory and 512 bytes of on-chip RAM. Its 8051-compatible data-memory organization includes the conventional 256-byte internal data-memory area, while the STC89xx family also provides additional RAM features depending on the exact device configuration. For the simulator, the conventional 8051 memory model should be kept clearly separated from any extended device-specific memory.

| Memory / Area | Size / Addressing | Purpose |
| --- | --- | --- |
| Program Flash | 8 KB on-chip | Stores program/instruction code. |
| Internal data RAM | 256-byte 8051 data space | Variables, register banks, bit-addressable area and general-purpose RAM. |
| Lower RAM | 00H–7FH | Direct and indirect addressing. |
| Register banks | 00H–1FH | Four banks, each containing R0–R7. |
| Bit-addressable RAM | 20H–2FH | 128 individually addressable bits. |
| Upper RAM | 80H–FFH | 128 bytes of upper RAM, accessed indirectly in the standard 8051 organization. |
| SFR space | 80H–FFH | Control/status registers, accessed by direct addressing. |
| Stack | Uses internal RAM | Stores return addresses and temporary stack data; controlled by SP. |

# 3. Internal RAM Organization

The lower 128 bytes of internal RAM are divided into three important regions. The first 32 bytes contain four register banks. The next 16 bytes are bit-addressable. The remaining 48 bytes are general-purpose RAM.

| Address Range | Size | Organization / Use |
| --- | --- | --- |
| 00H–07H | 8 bytes | Register Bank 0: R0–R7 |
| 08H–0FH | 8 bytes | Register Bank 1: R0–R7 |
| 10H–17H | 8 bytes | Register Bank 2: R0–R7 |
| 18H–1FH | 8 bytes | Register Bank 3: R0–R7 |
| 20H–2FH | 16 bytes | Bit-addressable RAM (128 bits) |
| 30H–7FH | 80 bytes | General-purpose RAM |

# 4. Register Banks and R0–R7

The 8051 architecture contains four register banks. Each bank has eight 8-bit registers named R0 through R7. Only one bank is active at a time. The RS1 and RS0 bits of the Program Status Word (PSW) select the active bank.

| RS1 | RS0 | Selected Bank | Address Range |
| --- | --- | --- | --- |
| 0 | 0 | Bank 0 | 00H–07H |
| 0 | 1 | Bank 1 | 08H–0FH |
| 1 | 0 | Bank 2 | 10H–17H |
| 1 | 1 | Bank 3 | 18H–1FH |

Simulator implication: the simulator should store all four banks but expose R0–R7 according to the current PSW.RS1/RS0 selection.

# 5. Main CPU Registers

| Register | Size | Address / Type | Function |
| --- | --- | --- | --- |
| ACC (A) | 8-bit | E0H, SFR | Accumulator used extensively for arithmetic, logic and data movement. |
| B | 8-bit | F0H, SFR | Used by MUL AB and DIV AB and can also be used as a general register. |
| PSW | 8-bit | D0H, SFR | Contains CPU status flags and register-bank selection bits. |
| SP | 8-bit | 81H, SFR | Stack Pointer; points to the current top of the stack. |
| DPTR | 16-bit | DPL=82H, DPH=83H | 16-bit data pointer for addressing data/external memory. |
| PC | 16-bit | CPU register | Program Counter; holds the address of the next instruction to fetch. |
| R0–R7 | 8-bit each | Internal RAM | Working registers in the currently selected register bank. |

# 6. Program Status Word (PSW)

PSW is an important SFR because it records arithmetic status and determines the active register bank.

| Bit | Name | Meaning |
| --- | --- | --- |
| 7 | CY | Carry flag; affected by arithmetic and some Boolean operations. |
| 6 | AC | Auxiliary Carry flag, mainly relevant to BCD arithmetic. |
| 5 | F0 | User-definable flag. |
| 4 | RS1 | Register-bank select bit 1. |
| 3 | RS0 | Register-bank select bit 0. |
| 2 | OV | Overflow flag. |
| 1 | F1 | User-definable flag. |
| 0 | P | Parity flag associated with the accumulator. |

# 7. Stack and Stack Pointer

The stack is located in internal data RAM. The Stack Pointer (SP) is an 8-bit SFR at address 81H. It identifies the current stack position. During PUSH operations the stack grows upward in internal RAM. CALL/ACALL instructions and interrupts also use the stack to preserve return addresses. POP and RET/RETI operations retrieve stack data as appropriate.

After reset, SP has the standard 8051 reset value of 07H.

A PUSH operation increments SP before storing the byte.

A POP operation reads the byte at SP and then decrements SP.

CALL/ACALL and interrupts push return information onto the stack.

The simulator should detect stack growth beyond the available internal RAM region.

# 8. Special Function Registers (SFRs)

SFRs are control and status registers located in the 80H–FFH address range. They control ports, timers, serial communication, interrupts and CPU functions. SFRs are accessed using direct addressing.

| SFR | Address | Main Purpose |
| --- | --- | --- |
| P0 | 80H | Port 0 latch / I/O control |
| SP | 81H | Stack Pointer |
| DPL | 82H | Low byte of DPTR |
| DPH | 83H | High byte of DPTR |
| PCON | 87H | Power-control functions |
| TCON | 88H | Timer 0/1 control and external interrupt control |
| TMOD | 89H | Timer 0/1 mode selection |
| TL0 / TL1 | 8AH / 8BH | Low timer/counter bytes |
| TH0 / TH1 | 8CH / 8DH | High timer/counter bytes |
| P1 | 90H | Port 1 latch / I/O control |
| SCON | 98H | Serial port control |
| SBUF | 99H | Serial data buffer |
| P2 | A0H | Port 2 latch / I/O control |
| IE | A8H | Interrupt enable control |
| P3 | B0H | Port 3 latch / I/O control |
| IP | B8H | Interrupt priority control |
| PSW | D0H | Program status word |
| ACC | E0H | Accumulator |
| B | F0H | B register |

# 9. Bit-Addressable Memory and SFRs

The internal RAM area 20H–2FH contains 128 individually addressable bits. This is useful for Boolean variables and bit-oriented instructions. In addition, selected SFRs are bit-addressable. A bit-addressable SFR is one whose SFR address is divisible by 8 in the standard 8051 map.

Bit-addressable RAM: byte addresses 20H–2FH.

This area provides 128 individual bits.

Important bit-addressable SFR examples include P0, TCON, P1, SCON, P2, IE, P3, IP and PSW.

The simulator should support both byte-level and bit-level access where the instruction set requires it.

# 10. DPTR and Data Addressing

DPTR is a 16-bit data pointer made from DPH and DPL. DPH contains the high byte and DPL contains the low byte. The STC89C52RC also supports a second data pointer through its device-specific DPTR switching mechanism. For a first version of the educational simulator, DPTR0 can be modeled as the primary 16-bit pointer, with DPTR1 added if the team implements the enhanced feature.

# 11. Simulator Data Model Recommendation

The following logical representation can be used when implementing the simulator. It keeps the CPU state separate from memory and makes register updates visible after each instruction.

| Component | Suggested Representation | What the UI Can Display |
| --- | --- | --- |
| Accumulator | 8-bit integer / byte | ACC value in hex and binary |
| B register | 8-bit integer / byte | B value |
| PSW | 8-bit integer + named flags | CY, AC, RS1, RS0, OV, P |
| PC | 16-bit integer | Current instruction address |
| SP | 8-bit integer | Stack pointer and stack contents |
| DPTR | 16-bit integer | DPH, DPL and combined address |
| R0–R7 | 4 × 8-byte banks | Current bank and all register values |
| Internal RAM | 256-byte array | Address, value and region |
| SFRs | Address-to-register mapping | Register name, address and current value |
| Stack | RAM-backed structure | Stack entries and SP position |

# 12. Example Memory/Instruction Trace

Example for the simulator: assume SP = 07H and instruction PUSH 30H is executed. The simulator should first increment SP to 08H and then copy the byte stored at RAM address 30H into RAM address 08H. The trace can show both operations so that students can understand stack behavior.

| Step | CPU / Memory Action | Expected State |
| --- | --- | --- |
| 1 | Before PUSH | SP = 07H; RAM[30H] = data |
| 2 | Increment SP | SP = 08H |
| 3 | Store source byte | RAM[08H] = RAM[30H] |
| 4 | After PUSH | SP = 08H; stack top contains source byte |

# 13. Key Points for Project Implementation

Maintain separate representations for PC, ACC, B, PSW, SP and DPTR.

Model the four register banks and use PSW.RS1/RS0 to select the active bank.

Represent RAM by address so instructions can read and write individual bytes.

Represent SFRs by their addresses and connect them to peripheral/control behavior later.

Implement stack operations using SP and internal RAM.

Display register and memory state after each instruction for educational tracing.

Clearly distinguish program memory (Flash/instruction storage) from data memory (RAM/SFRs).

Add device-specific STC89C52RC features only after the core 8051 model is working.

# 14. Conclusion

The register and memory organization of the STC89C52RC forms the foundation for the simulator's CPU-state and memory modules. Correct modeling of the register banks, PSW, accumulator, DPTR, stack, internal RAM and SFRs will allow the simulator to execute instructions and visibly demonstrate how the microcontroller changes state. This documentation can therefore serve as the Week 1 technical reference for the register/memory portion of the project.

# 15. References

STC Micro Technology Co., Ltd., STC89C52RC Datasheet / STC89C52RC product documentation.

Intel MCS-51 / 8051 architecture documentation and standard 8051 instruction-set references.

STC Micro Technology product page: STC89C52RC series.